11.4.2 Propeller Malfunctions. A propeller malfunction

may be caused by electrical or synchrophaser

malfunction, or hydraulic malfunction, and will be

indicated by one of the following conditions:

1. Propeller LOW OIL light or visible oil leak.

2. Overspeed or underspeed.

3. Rpm surge or fluctuation.

4. Failure of propeller to feather.

Note

A tachometer generator failure will give a

false indication of propeller failure when

underspeeding or fluctuations occur. Refer

to Tachometer Generator Failure, paragraph

11.4.10.5.
