2.4.4.4 Prop Resynchrophase Switch. The

prop resynchrophase switch (see Figure 2-19) is a

two-position (NORMAL, RESYNC) toggle switch

located on the flight control pedestal. The switch is

spring-loaded and shear-wired to the NORMAL position.

The prop resynchrophase switch shall not be

positioned to RESYNC except when performing the

propeller re-indexing procedure and should be used

only for correcting an out-of-sync or off-speed condition

by performing the complete re-indexing procedure.

These airplanes will have a decal installed above the

prop resynchrophase switch on the flight control

pedestal stating SOLID STATE SYNCHROPHASER

INSTALLED. REFER TO APPROVED FLIGHT

MANUAL FOR OPERATION. Power is supplied

through the SYNCHROPHASER 28V essential dc

circuit breaker located on the copilot side circuit breaker

panel.


CAUTION

The PROP RESYNCHROPHASE switch

shall not be positioned to RESYNC except

when performing the propeller reindexing

procedure and should be used only for

correcting an out-of-sync or off-speed 

condition by performing the complete

reindexing procedure.
[image: image1.jpg](%
e S \

SOLID STATE s?NcHRorﬂAszR
INSTALLED. REFER TO ‘;z
APPROVED FLIGHT MANUAL
FOR. OPERATION e

@ UHF ANT

SEL NO.|




