11.4.2.5 Propeller Fails to Feather. If the

propeller rotation continues after feather has been

initiated:

1. Attain 150 KTAS (if possible) (P).

Note

Slow aircraft to the minimum safe airspeed,

but not less than two-engine air minimum

control speed (135 KIAS minimum).

2. FEATHER and AIRSTART, EMER FEATHER,

FEATHER PUMP MOTOR circuit breakers —

Checked In (FE).

3. Feather Override Button — Hold in for 30 seconds,

Pull Out (FE).

If propeller rotation continues:



WARNING

Restore oil only if there is no indication of a

fire.

4. Fire handle — Reset (CP).

5. OIL SHUTOFF VALVE circuit breaker — Pull

(FE).

6. Fire handle — Pull (CP).



WARNING

If a go-around is attempted, follow Go-around

with One or Two Engines Inoperative,

paragraph 11.8.2. Go-around with a

windmilling propeller may be marginal.

Note

If the propeller does not feather, a landing

can be made with a windmilling propeller;

however, the drag and yawing tendency will

be greater than with a feathered propeller,

and excessive rpm and noise may be experienced.
