11.4.2.2 In-Flight Propeller Malfunctions



WARNING

If uncontrolled overspeed (above 105-percent

rpm) occurs, reduce airspeed as rapidly as

possible to the speed at which safe control of

the aircraft or propeller can be maintained but

not less than two-engine air minimum control

speed (at least 135 KIAS). Do not adjust the

throttle position for the affected engine before

the malfunction is analyzed.

11.4.2.2.2 Rpm Outside Allowable Limits With

LOW OIL Light Illuminated

1. Place propeller governing control switch to

MECH GOV.

a. If rpm stabilizes within allowable limits,

perform Engine Shutdown Procedure,

paragraph 11.1.2.



WARNING

If continued operation of the affected propeller

is required in the interest of safety, it is

permissible to defer engine shutdown until

landing is assured, provided that moderately

high power is maintained.

2. If rpm does not stabilize within allowable limits,

perform Pitchlock Check Procedure, paragraph

11.4.2.3.
