2.4.4.3 SYNCHROPHASE MASTER Switch.

The SYNCHROPHASE MASTER switch (see Figure

2-19) is located on the flight control pedestal. This

three-position (ENG 2, OFF, ENG 3) toggle switch

controls the operation of the synchrophase system and

selects the engine to be used as the master. When the

switch is in the ENG 2 position, the No. 2 engine is

selected as the master and the other propeller phase

angles are referenced to this engine. When the switch is

in the OFF position and the propeller governor control

switches are in the NORMAL position, the synchrophasing

system is turned off and the propellers operate in normal 

governing. If the propeller governor control switches are 

in the MECH GOV position, the propellers

operate in mechanical governing. When the switch is in

the ENG 3 position, the No. 3 engine is the master and

the other propeller phase angles are referenced to this

engine.

[image: image1.jpg](%
e S \

SOLID STATE s?NcHRorﬂAszR
INSTALLED. REFER TO ‘;z
APPROVED FLIGHT MANUAL
FOR. OPERATION e

@ UHF ANT

SEL NO.|




