2.4.1.5 Spinner Assembly. The spinner assembly,

which improves the aerodynamic characteristics of

the propeller assembly, encloses the dome, barrel, and

control assemblies. It consists of a front section, a rear

section, and a nonrotating afterbody assembly. Cooling

air is admitted through an air inlet at the front of the

spinner and passes over the dome assembly, barrel

assembly, and control assembly fins, and exhausts

through vents in the engine nacelle.
