2.4.4.7 Feather Override Buttons. Four

FEATHER OVERRIDE buttons are located on the

copilot side shelf (see Figure 2-18) to provide a means

of manually stopping the propeller auxiliary pump.

When a condition lever is moved to FEATHER,

electrical circuits are completed to the feather solenoid

valve, propeller auxiliary pump, pressure cutout switch,

and a holding circuit to the feather override button. If a

FEATHER OVERRIDE button is manually pulled after

a condition lever is placed in FEATHER, the electrical

circuits for propeller feathering are interrupted. If a

FEATHER OVERRIDE button does not pop out after

the feather cycle is complete, it should be pulled

manually to prevent damage to the propeller auxiliary

pump. The holding coil of the override button receives

28-Vdc power from the essential dc bus through the

FEATHER & AIRSTART or EMER FEATHER circuit

breaker on the copilot side circuit breaker panel.

[image: image1.jpg]A BRI
COPILOTS siDE
PANEL LIGHTS

3
PROPELLER FEATHER OVERRIDE

ey

'—'\Pﬂgl’ﬂlﬂ! GOVERNOR Coni

OFF BRIGH

E0GE
OVERHEAD PANEL LIGHTS
OFF BRIGHT
FLoop
OVERHEAD PANEL LIGHTS

OFF

BRIGHT
INSTRUMENT

COPILOTS 1newn.





